System Requirements Specification Test Coverage Example

Prepared by: GSR

Generated on: 8/13/2012

Table of Contents

Automated Meter Reader System Requirements	3

1	Introduction	3

1.1	Purpose of the Document	3

2	General Description	3

2.1	Functions and Purpose	3

2.2	Environmental Considerations	4

3	System Requirements	4

3.1	Functional Requirements	4

3.1.1	Handheld device	4

3.1.2	Meter Interface Unit	5

3.1.3	Fixed Network Automated Meter Reading System	6

3.1.4	Control Computer and Related Hardware	7

3.2	Non-Functional Requirements	7

3.2.1	Control Computer	7

3.2.1.1	Application Server	8

3.2.1.2	Database Server	8

3.2.2	Handheld Unit	9

3.2.3	Meter Interface Unit	9

3.2.4	Assumptions and Dependencies	10

Automated Meter Reader System Requirements

| Baseline: | Current |
| --- | --- |
| Last Modified On: | 2012-08-02 |

Total requirements: 51	Requirements without test case(s): 48

Requirements coverage in module: 94% not covered

# Introduction

## Purpose of the Document

| AMR-SR-31 | This document describes the specific functionality of the Automated Meter Reader system. The system is currently available with a handheld collection device. The mobile and fixed network methods of data collection are outside the scope of this system. | No linked test case! |
| --- | --- | --- |

# General Description

## Functions and Purpose

| AMR-SR-32 | The AMR system is used to determine water service / consumption for the more than 79,000 meter connections to residential, commercial and industrial customers inside a 72 square mile area. | No linked test case! |
| --- | --- | --- |

| AMR-SR-111 | In handheld AMR, a meter reader carries a handheld computer with a built-in or attached receiver/transceiver (radio frequency or touch) to collect meter readings from an AMR capable meter. This is sometimes referred to as "walk-by" meter reading since the meter reader walks by the locations where meters are installed as they go through their meter reading route. Handheld computers may also be used to manually enter readings without the use of AMR technology as an alternate but this will not support comprehensive data which can be accurately read using the meter reading electronically. | No linked test case! |
| --- | --- | --- |

| AMR-SR-121 | <AMR Artist Rendition> | No linked test case! |
| --- | --- | --- |

## Environmental Considerations

| AMR-SR-34 | The control computer shall be capable of operating in a normal office environment. | No linked test case! |
| --- | --- | --- |

# System Requirements

## Functional Requirements

### Handheld device

| AMR-SR-39 | The handheld device shall provide for the means for the meter reader to manually enter a meter reading. | Verified by test case 33 Verified by test case 27 |
| --- | --- | --- |

| AMR-SR-37 | The handheld device shall interfaces with the city's backoffice software. | No linked test case! |
| --- | --- | --- |

| AMR-SR-40 | The handheld device shall allow for programming of a defined route, advancing to the next meter on the route as the meter reader moves through the route. | No linked test case! |
| --- | --- | --- |

| AMR-SR-41 | The handheld device shall have the ability to search for accounts by Last Name, Service Address, Meter Number, and Unread Meters. | No linked test case! |
| --- | --- | --- |

| AMR-SR-42 | The handheld device shall have a screen capable of displaying the number of accounts that have been read and unread. Display information shall include: total number of accounts in collection route, number of read accounts, number of unread accounts, the address of each account. For completed (read) accounts, the display information shall include: the date and time of the last reading, summary of usage data, and the id of handheld reading device. | No linked test case! |
| --- | --- | --- |

| AMR-SR-43 | The handheld device shall allow the meter reader to enter information about meters relocated on a particular route. | No linked test case! |
| --- | --- | --- |

| AMR-SR-44 | Information captured via the handheld device shall be downloadable via either cable hookup or wireless signal. | No linked test case! |
| --- | --- | --- |

| AMR-SR-46 | The handheld device shall be able to recharge using solar power. | No linked test case! |
| --- | --- | --- |

| AMR-SR-80 | The handheld device shall include a leak indicator. | Verified by test case 26 |
| --- | --- | --- |

| AMR-SR-77 | The handheld device shall provide a means to automatically (electronically) read the meter. | No linked test case! |
| --- | --- | --- |

| AMR-SR-78 | Individual meter usage data and leak diagnostic data, when successfully uploaded to the handheld device, shall be immediately available for display on the handheld device. | No linked test case! |
| --- | --- | --- |

| AMR-SR-82 | The handheld device shall display the following data for leakage: timestamp, meter ID | No linked test case! |
| --- | --- | --- |

### Meter Interface Unit

| AMR-SR-54 | The meter interface unit shall operate using walk-by, mobile (vehicle-based), and mesh network collection platforms. | No linked test case! |
| --- | --- | --- |

| AMR-SR-49 | The meter interface unit shall support all data collection functions (data reading, time-triggered operation, and management) of the AMR system. | No linked test case! |
| --- | --- | --- |

| AMR-SR-55 | The meter interface unit shall employ two-way communications down to the endpoint making it possible for operators to 'push' interval data requests, firmware updates, new capabilities and updated monitoring schedules via the network. | No linked test case! |
| --- | --- | --- |

| AMR-SR-50 | The meter interface unit shall be compatible with the existing meter models in use for the area covered by this project. | No linked test case! |
| --- | --- | --- |

| AMR-SR-52 | The meter interface unit shall be powered by a replaceable long lasting battery (lithium or other). | No linked test case! |
| --- | --- | --- |

| AMR-SR-56 | The meter interface unit shall capture usage data hourly and store this consumption data for up to 365 days. This hourly consumption data is considered usage profile data. | No linked test case! |
| --- | --- | --- |

| AMR-SR-65 | Meter usage data and leak diagnostic data shall be retrievable on demand from any meter interface via the network or a handheld. | No linked test case! |
| --- | --- | --- |

| AMR-SR-57 | When connected to a fixed network, the meter interface unit shall 'wake up' and communicate for 4 seconds every 30 minutes, synchronizing all clocks and configuration information. Between these transmissions, the unit remains in a low power state, conserving battery life. | No linked test case! |
| --- | --- | --- |

| AMR-SR-68 | The meter interface shall detect water leaks and record leak status with the account data. | No linked test case! |
| --- | --- | --- |

| AMR-SR-83 | The meter interface unit shall constantly monitor pressure to determine leakage. | No linked test case! |
| --- | --- | --- |

### Fixed Network Automated Meter Reading System

| AMR-SR-103 | The system shall have a permanently installed network to capture meter readings | No linked test case! |
| --- | --- | --- |

| AMR-SR-104 | The system shall collect transmissions of meter readings from AMR capable meters and get the data to a central computer without a person in the field to collect it. | No linked test case! |
| --- | --- | --- |

| AMR-SR-105 | The systems shall forward a reading from a more remote area back to a main collector without actually storing it. | No linked test case! |
| --- | --- | --- |

| AMR-SR-110 | The system shall consist of a series of antennas, towers, collectors, repeaters, or other permanently installed infrastructure to collect transmissions of meter readings from AMR capable meters and get the data to a central computer without a person in the field to collect it. | No linked test case! |
| --- | --- | --- |

### Control Computer and Related Hardware

| AMR-SR-108 | The city will require a two server system, application and data storage. | No linked test case! |
| --- | --- | --- |

| AMR-SR-109 | The control computer shall be capable of operating in a normal office environment using normal office power supply. Uninterruptible Power Supply; the control computer shall include an Uninterruptible Power Supply (UPS) of at least 4 hours. | No linked test case! |
| --- | --- | --- |

## Non-Functional Requirements

### Control Computer

| AMR-SR-119 | The control computer shall be capable of operating in a normal office environment. | No linked test case! |
| --- | --- | --- |

#### Application Server

| AMR-SR-97 | Application server with the following minimum specifications: - Two dual-core processors with minimum 3.00 Ghz processor and 1333 Frontside Buss (FSB) - Memory - 8Gb fully buffered DIMM - Storage controller - 1st hard drive - 146 GB Hot Plug, 2.5 SAS, 10,000 rpm Hard Drive - 2nd hard drive - 146 GB Hot Plug, 2.5 SAS, 10,000 rpm Hard Drive - Mirrored - Network card - 2 Embedded Multifunction Gigabit Network Adapters - Power supply - 700w Hot Plug Power Supply - Redundant power supply - Redundant fans - Multimedia drive - DVD+RW 8x drive - Server management Warranty - 3 years parts on-site labor, next business day | No linked test case! |
| --- | --- | --- |

#### Database Server

| AMR-SR-100 | Database server with the following minimum specifications: - Two dual-core processors with minimum 3.00 Ghz processor and 1333 Frontside Bus (FSB) - Memory - 12GB fully buffered DIMM - Storage controller - 1st hard drive - 146 GB Hot Plug, 2.5 SAS, 10,000 rpm Hard Drive - 2nd hard drive - 146 GB Hot Plug, 2.5 SAS, 10,000 rpm Hard Drive - Mirrored - 3 Hard drives configured in RAID 5 configuration - 146 GB Hot Plug, 2.5 SAS, 10,000 rpm Hard Drive, 1 - Hot Swap Spare - 146 GB Hot Plug, 2.5 SAS, 10,000 rpm drives. - Network card - 2 Embedded Multifunction Gigabit Network Adapters - Power supply - 700w Hot Plug Power Supply - Redundant power supply - Redundant fans - Multimedia drive - DVD+RW 8x drive 17 - Server management Warranty - 3 years parts on-site labor, next business day | No linked test case! |
| --- | --- | --- |

### Handheld Unit

| AMR-SR-114 | The handheld unit shall be no larger than 25cm x 18cm x 1cm. | No linked test case! |
| --- | --- | --- |

| AMR-SR-115 | The handheld unit shall have a mass no greater than 2.25kg. | No linked test case! |
| --- | --- | --- |

| AMR-SR-116 | The handheld unit shall function in environments from -5 degree C through +50 degree C. | No linked test case! |
| --- | --- | --- |

| AMR-SR-117 | The handheld unit shall function in environments with 99% ambient humidity. | No linked test case! |
| --- | --- | --- |

| AMR-SR-118 | The handheld unit's external case shall have no sharp edges and no pointed corners (minimum radius 2mm). | No linked test case! |
| --- | --- | --- |

| AMR-SR-122 | The handheld unit shall remain fully functional following a drop onto concrete from a height of one metre onto any edge, face or corner. | Verified by test case 25 Verified by test case 36 |
| --- | --- | --- |

| AMR-SR-123 | The handheld unit shall have a design life of 5 years | No linked test case! |
| --- | --- | --- |

| AMR-SR-124 | The handheld unit shall have a MTBF of 10000 hours | No linked test case! |
| --- | --- | --- |

### Meter Interface Unit

| AMR-SR-126 | The meter interface unity shall have a MTBF of 50000 hours | No linked test case! |
| --- | --- | --- |

| AMR-SR-127 | The meter interface unit shall have only tamper-proof fixings externally (specification TBD) | No linked test case! |
| --- | --- | --- |

| AMR-SR-128 | The meter interface unit shall operate at temperatures from -40C to +85C | No linked test case! |
| --- | --- | --- |

### Assumptions and Dependencies

| AMR-SR-62 | The control computer shall be capable of operating in a normal office environment using normal office power supply. Uninterruptible Power Supply; the control computer shall include a UPS of at least 4 hours. | No linked test case! |
| --- | --- | --- |

| AMR-SR-45 | The handheld device shall provide for a minimum of 16MB of memory in order to support field data collection. | No linked test case! |
| --- | --- | --- |

| AMR-SR-53 | The meter interface unit shall be able to store data into nonvolatile RAM which does not need power supply to maintain memory. | No linked test case! |
| --- | --- | --- |

| AMR-SR-61 | The handheld device shall support usage to determine water service / consumption for the more than 78,000 meter connections to residential, commercial and industrial customers inside a 72 square mile area. | No linked test case! |
| --- | --- | --- |