Updating Xseries firmware.

- This process involves rebooting the Xseries servers, it can only be done while the system is not available for IO.

- All the 3550 and 3650 Xseries servers need to run this process, all of the drawers can be updated at the same time, if enough disks are available.

- Place IA_FMW_v1.2 disk in CD/DVD reader of the server to be updated.

- At command issue the command “sudo /sbin/reboot.”

- The server will reboot, and automatically load the firmware updates from the disk.

  - Allow between 10 – 30 minutes for updates to complete.

- At the review screen press q then Enter to quit, select yes for “are you sure” prompt.

- At next screen press q then Enter again to quit out.

- At this point the system reboots and ejects the disk, if the disk fails to eject, simply eject the disk at power up.

- Remove the disk and close the disk tray.

- Firmware updates are now complete.