When upgrading the secondary appliance in an Enhanced Remote Mirroring configuration to IBM Information Archive version 1.2.0 or later, you must change the Ethernet ports that are used by the disk controllers.

Complete the following steps on the secondary appliance:

- Open the DS Storage Manager interface by issuing the following command on the management console server: sudo /usr/bin/SMclient

- In the DS Storage Manager interface, click the Devices tab.

  - If any of the storage subsystems in the appliance are not shown, complete the following steps:

    - For each storage subsystem that is not shown:

      - In the upper left corner of the page, click Edit and select Add Storage Subsystem.

      - Click Out-of-Band Management and enter the controller addresses for the subsystems that are not shown.

Restriction: You must add the controllers one at a time.

| D1A Disk Controller #1b | 172.30.0.100/ 172.30.0.101 |
| --- | --- |
| D1A Disk Controller #2b | 172.30.0.102/ 172.30.0.103 |
| D1A Disk Controller #3b | 172.30.0.104/ 172.30.0.105 |

      - Click Add.

      - You are prompted to add more controllers. Choose the appropriate response.

    - Close the Add Storage Subsystem page (or click No when prompted to add another storage subsystem).

- For each D1A Disk Controller in the appliance, complete steps 4-13 below, where “x” is the subsystem number (1, 2, or 3). Start with controller 1.

- Move the Ethernet cable in controller A port 2 to controller A port 1 for this 2231 D1A controller.

D1 D2

H2 H1

H1 H2

E1 E2

D2 D1

E2  E1

H1 H2

Controller A

Controller B

Move Ethernet Cable From E2            to E1

- On the Devices page of the DS Storage Manager interface, right-click iastoragexb (the controller for which you relocated the cable) and select Manage Storage Subsystem.

  - Click No when you are prompted to set a password.

  - Click the Physical tab on the ‘iastoragexb’ page.

  - Right-click controller A and select Configure > Ethernet Management Ports.

  - From the drop-down list at the top of the page, select controller A port 1 (if it is not selected already).

- Change the value in the IP and Subnet Mask field for controller A Port 1 to the appropriate address as listed in the following table:

| Controller | IP Address | Subnet Mask |
| --- | --- | --- |
| D1A Disk Controller #1b | 172.30.0.100 | 255.255.248.0 |
| D1A Disk Controller #2b | 172.30.0.102 | 255.255.248.0 |
| D1A Disk Controller #3b | 172.30.0.104 | 255.255.248.0 |

- Select controller A port 2 from the drop-down list.

- Change the value in the IP and Subnet Mask field for controller A Port 2 to the appropriate address as listed in the following table:

| Controller | IP Address | Subnet Mask |
| --- | --- | --- |
| D1A Disk Controller #1b | 192.168.251.100 | 255.255.255.0 |
| D1A Disk Controller #2b | 192.168.251.102 | 255.255.555.0 |
| D1A Disk Controller #3b | 192.168.251.104 | 255.255.255.0 |

  - Click OK.

  - You are prompted to rediscover storage. Click Yes. This process can take some time to complete.  A ‘Working’ message is shown while the process runs.

  - When the process completes, verify the IP address changes by selecting Configure > Ethernet Management Ports on the Physical page.

- Move the cable in controller B port 2 to controller B port 1 for this controller.

D1 D2

H2 H1

H1 H2

E1 E2

D2 D1

E2  E1

H1 H2

Controller A

Controller B

Cable From E2

to E1

- Right-click controller B and select Configure > Ethernet Management Ports.

  - From the drop-down list at the top of the page, select controller B port 1 (if it is not selected already).

- Change the value in the IP and Subnet Mask field for controller B Port 1 to the appropriate address as listed in the following table:

| Controller | IP Address | Subnet Mask |
| --- | --- | --- |
| D1A Disk Controller #1 | 172.30.0.101 | 255.255.248.0 |
| D1A Disk Controller #2 | 172.30.0.103 | 255.255.248.0 |
| D1A Disk Controller #3 | 172.30.0.105 | 255.255.248.0 |

- Select controller B port 2 from the drop-down list.

- Change the value in the IP and Subnet Mask field for controller B Port 2 to the appropriate address as listed in the following table:

| Controller | IP Address | Subnet Mask |
| --- | --- | --- |
| D1A Disk Controller #1 | 192.168.251.101 | 255.255.255.0 |
| D1A Disk Controller #2 | 192.168.251.103 | 255.255.555.0 |
| D1A Disk Controller #3 | 192.168.251.105 | 255.255.255.0 |

  - Click OK.

  - You are prompted to rediscover storage. Click Yes. This process can take some time to complete.  A ‘Working’ message is shown while the process runs.

  - When the process completes, verify the IP address changes by selecting Configure > Ethernet Management Ports on the Physical page.

  - Close the page and close the Manage Storage Subsystem page. If the starting page of the DS Storage Manager interface is not shown, locate that page.

- Repeat steps 4-13 for each of the controllers in the secondary appliance (up to 3 controllers).

Tip: If any of the mirrored controllers become suspended, see the instructions for resuming mirroring in the IBM Information Archive Service Guide.

- For each storage subsystem in the secondary appliance, complete the following steps, where x is the subsystem number:

  - In the DS Storage Manager interface, click the Devices tab.

  - On the Devices page, right-click iastoragexb and select Remove > Storage Subsystem.

  - When prompted to verify the change, click Yes.

- Re-add the storage controllers, following the instructions in step 2 of this procedure.

- Log on to the management console server with the iaadmin user ID.

- Issue the following command:

sudo /opt/tivoli/tiam/bin/ia_setup_storage.sh

Tip: The setup process can take a few minutes to complete, depending upon the number of storage subsystems in the appliance.

- Verify that the storage subsystems are shown in the IBM Systems Director interface:

    - Log on to the Information Archive administrative interface. In the navigation tree, expand Information Archive Management and click Service Tools. Click Open IBM Systems Director. Log on to IBM Systems Director using the iaadmin user ID.

    - Expand the IBM Systems Director navigation tree and click Inventory > System Discovery.

    - Click Manage Discovery Profiles.

    - Select Default IA storage discovery and click Run.

    - Select Run now and click OK.

    - Click Navigate Resources.

    - In the Groups table, click All Systems. Verify that all of the storage subsystems are shown. It can take a few minutes for the discovery process to complete.