If the return status for any of the commands in Steps 1 to 3 review the “shell_recover_session.log” located in the C:\RecoveryTool\ Folder to determine the cause of the failure, unless noted otherwise. Also any “Type” errors that are seen can be ignored at this time as this can be caused by the symbols a controller can print out during a controller reboot.

- Stop Controller Reboots

  - Extract the 90.00.X5.XX to C:\RecoveryTool\ Folder

  - Connect the PC/Laptop to the A controller via a serial cable

  - Open up a Windows Command prompt by clicking Start -> Run and entering cmd and pressing enter

  - At the command prompt type: cd C:\RecoveryTool\Script\

  - Enter: shell_recover log_note <StorageSubsystem Name>

  - Enter: shell_recover –c COMX check_serial
NOTE: If the shell password has been changed the –p <password> option will need to be added in the first command.

  - Enter: shell_recover capture_reboot

  - Enter: shell_recover stop_reboot
It the 7-Segment LED on the controller do not change to AL re-run the command

  - Disconnect the serial cable from the controller and connect it to its Alternate

  - Enter: shell_recover check_serial

  - Enter: shell_recover stop_reboot
It the 7-Segment LED on the controller do not change to AL re-run the command

- Data Collection

  - Start the ftpserver that is bundled with either the Database Verification Utility or ftpserver bundled with the Recovery Utility

  - Enter: shell_recover set_vki

  - Enter: shell_recover collect_trace
This step can take 30 minutes or longer to complete

  - Enter: shell_recover clear_vki

  - Disconnect the serial cable from the controller and connect it to its Alternate

  - Enter: shell_recover set_vki

  - Enter: shell_recover collect_trace

  - Enter: shell_recover load_network

  - Enter: shell_recover config_ftp –f  <FTP_Server_IP>

  - Enter: shell_recover do_fsck

  - Enter: shell_recover collect_debug
This step can take 15 to 30 Minutes

  - Enter: shell_recover dbm_verify
If this step fails check the LED on the controller. If the LED is “AL” then re-run step 2h and then re-run this step

  - Enter: shell_recover clear_vki

- Clearing Configuration

  - Enter: shell_recover clear_config

  - Enter: shell_recover cleanup

  - Wait for the Controllers 7 Segment LED to display 85

  - Disconnect the serial cable from the controller and connect it to its Alternate

  - Power Down and pull the Controller that the configuration was just cleared on and power up the controller module

  - Enter: shell_recover clear_config

  - Enter: shell_recover cleanup

  - Wait for the LED on the controller to go to 85 and re-insert the alternate controller

  - The A controller may come up in a failed state and should be set online via the GUI.

- Creating Configuration Scripts

  - Gather the recovery profiles from the last known PC to Manage the storage system. The recovery profiles are located in the following directory on a Windows Host
C:\Program Files\IBM_DS4000\client\data\recovery\
The recovery profiles are the files with the .csv extension.

  - Move the Recovery Profiles to a folder on the desktop of the host with the Recovery Tool installed.

  - If an old Storage Subsystem Configuration file exists copy it to the Desktop.

  - Open up the “RecoveryTool.bat” located in the C:\RecoveryTool\

  - From the “Configuration Recovery Tool” window click Browse and select the folder on the desktop with the recovery profiles. NOTE: The Recovery Profiles will now show up in the Browse Window.

  - Select the Storage Subsystem from the “Select storage array” to select the configuration to re-create

  - If an old Storage Subsystem Configuration file exists click the Browse button next to the Configuration file location in the Configuration Recovery Tool Window and select the old configuration file.

  - Click on the Browse button for the “Recovery file location.” Then select a directory to where the recovery scripts will be created.

  - Click Start

- Loading Storage Subsystem

  - Re-Add the Storage Subsystem using either in-band or out of band

  - Right click on the Storage Subsystem and click Execute Script.

  - Click File -> Load Script and browse to location selected in Step 4h and click on the “……..LogicalDriveCfg.script”

  - Click Tools -> Verify and Execute

  - Close down the script editor window

- Re-Create Logical Drive WWNs if attached to a SVC

  - Copy the “………._LogicalDrives.csv” file located in the selected directory from Step 4h to the C:\RecoveryTool\Script directory

  - Switch to the Command prompt opened up in Step 2

  - Enter: shell_recover restor_wwns <………._LogicalDrives.csv>

  - Enter: shell_recover reboot_alt

  - Enter: shell_recover reboot_primary

- Re-Create Host to Logical Drive Mappings

  - Switch to Storage Manager

  - Right click on the Storage Subsystem and click Execute Script.

  - Click File -> Load Script and browse to location selected in Step 4h and click on the “…….._MappingCfg.script”

  - Click Tools -> Verify and Execute

  - Close down the script editor window

- Check Data from the Host

  - Bring the host and verify that the data is accessible.